RHL-TH130 Digital Metal Hardness Tester

### Advantages

- Large screen(128×64 dot matrix LCD), showing all functions and parameters.

- Test at any angle, even upside down.

- Wide measuring range. It can measure the hardness of all metallic materials. Direct display of hardness scales HRB, HRC, HV, HB, HS, HL and three types of strength values immediately.

- Seven impact devices are available for special application. Automatically identify the type of impact devices.

- Large capacity memory could store 1000 groups (Relative to average times32～1 ) information including single measured value, mean value, testing data, impact direction, impact times, material and hardness scale etc.

- Upper and lower limit can be preset. It will alarm automatically when the result value exceeding the limit.

- Battery information indicates the rest capacity of the battery and the charge status.

- User calibration function.

- USB port with the PC humanity multi-functions data proceeding software.

- Original imported high speed thermal printer support the immediate printing function. It can save data permanently.

- NI-MH rechargeable battery as the power source. Charge circuit integrated inside the instrument. Continuous working period of no less than 150 hours (EL off and no printing).

- Auto power off to save energy.

- Excellent after-sale service system for high quality products---two years’ guarantee and all life maintenance. Easy to buy and comfortable to use.

Main Application

- Die cavity of molds

- Bearings and other parts

- Failure analysis of pressure vessel, steam generator and other equipment

- Heavy work piece

- The installed machinery and permanently assembled parts

- Testing surface of a small hollow space

- Material identification in the warehouse of metallic materials

- Rapid testing in large range and multi-measuring areas for large-scale work piece

## Technical Specifications

- Measuring range：HLD（170～960）HLD

- Measuring direction：360°

- Hardness Scale：HL、HB、HRB、HRC、HRA、HV、HS

- Display：dot matrix LCD，128×64 dots

- Data memory： 500 groups max.（relative to impact times 32～1）

- Printing paper: width is （57.5±0.5）mm, diameter is 30mm

- Battery pack: 6V NI-MH

- Battery charger: 9V/500mA

- Continuous working period：about 150 hours（With backlight off, no printing）

- Communication interface：USB1.1

- Outline dimensions：212mm×80mm×32mm

### Configuration

|  | No. | Item | Quantity | Remarks |
| --- | --- | --- | --- | --- |
| Standard Configuration | 1 | Main Unit | 1 |  |
|  | 2 | D type impact device | 1 | Without cable |
|  | 3 | Standard test block | 1 |  |
|  | 4 | Cleaning brush (I) | 1 |  |
|  | 5 | Small support ring | 1 |  |
|  | 6 | Battery Charger | 1 | 9V 500mA |
|  | 7 | Paper for printing | 1 |  |
|  | 8 | Manual | 1 |  |
|  | 9 | DataPro software | 1 |  |
|  | 10 | Communication cable | 1 |  |
|  | 11 | Instrument case | 1 |  |
|  | 12 | Cleaning brush (II) | 1 | For use with G type impact device |
|  | 13 | Other type of impact devices and support rings |  | Refer to Table 3 and Table 4 in the appendix. |